Fully Automated DDS Sweep Generator Measurement System

Program Notes by Dr. Sam Green W0PCE

After this article went to press, I generated separate versions of the DDS programs for the original NJQRP DDS-30 that operates to 30 MHz and the new NJQRP DDS-60 that operates to 60 MHz.  In the download package, you will find SWDDS30, SWcenter30, SSW30, and LSW30 for the DDS-30 and SWDDS60, SWcenter60, SSW60, and LSW60 for the DDS-60 in place the SWDDS, SWcenter, SSW, and LSW described below.  Software for the new DDS will not run the original DDS because the least significant bit in the phase word must be different.  All programs terminate with ‘Control-C’.  “Pause” and “Enter” freeze and restart the display.

ADC.EXE

ADC will start from a command line in a DOS window or by double clicking.

A usage statement appears that tells you the command line parameters to enter to decrease the resolution to 12 bits for faster measurements or increase it to 14 bits for more precise but slower measurements.

Usage:	ADC 12	12 bit resolution (plus sign)

ADC 13	13 bit resolution default

ADC 14	14 bit resolution but slow

When running, the program displays

- the elapsed time for each measurement and

- the voltage measured at Input 1

DAC.EXE

DAC will start from a command line only.  It requires at least one parameter to function.  With no parameters following the command, the program displays a usage statement that tells you proper syntax.

Usage: ADC Vstart [Vend] [#steps] [dwell]

Constant output for single argument

Sweeps for two arguments with Vstart less than Vend

and both between -2470 to +2470 mV

in integer mV with no decimal

and dwell per step in ms

With a single parameter, the output is a constant voltage and the program ends.  With two parameters, the output voltage sweeps from the first entry which must be the most negative value to the second entry that represents the most positive value, and repeats.  The possible range is about -2.5V to +2.5V.  Enter start and end values in integer millivolts as parameters following the command to select values within this range.

When running, the program displays either the constant value or

- the start voltage,

- the end voltage,

- the number of steps,

- the count increment of the number the program writes to the D/A Converter, and

- the instantaneous value of the output voltage

SWDDS.EXE

SWDDS will start from a command line or by double clicking.  The program displays a usage statement that tells you the proper syntax.

Usage: SWDDS [start] [end] [#steps]

With no parameters, SWDDS runs default settings to sweep from 1 MHz to 10 MHz in 4096 steps.

The numeric keypad allows coarse increases and decreases of start and end frequencies and number of steps.  These choices make sense if you consider the positions of the keys.

‘7’	doubles the start frequency to values less than 90% of the end frequency, or

raises it by 10% if the result would be higher than 90% of the end frequency.

‘1’	halves the start frequency

‘9’	doubles the end frequency with an upper limit of 30 MHz

‘3’	halves the end frequency to values greater than 110% of the start frequency, or

decreases it by 11% if the result would be lower than 110% of the start frequency.

‘8’	halves the number of steps

‘2’	doubles the number of steps with an upper limit of 4096

Only these keys change operating parameters.

The D/A Converter resolution limits the number of steps to 4096.  The DDS is able to provide resolution to a small fraction of a Hertz if you eliminate the step count limit and the minimum step size, which in turn defeats the D/A Converter analog sweep below a step count of one.

When running, the program displays

- the start frequency, the end frequency,

- the number of steps,

- the incremental DDS step count,

- the corresponding incremental frequency change,

- the D/A Converter sweep limits in hexadecimal numbers,

- the instantaneous output frequency,

- the incremental D/A Converter step count, and,

- the instantaneous word the program writes to the D/A Converter

I use integer math because the DDS, ADC, and DAC must receive binary numbers.  At low frequencies and small frequency steps, the integer math breaks down due to rounding error.  Do not expect to hit the number keys endlessly without some problems.

SWCENTER.EXE

SWCENTER will start from a command line or by double clicking.  The program displays a usage statement that tells you the proper syntax.

Usage: SWCENTER [CenterFreq] [SweepWidth] [#steps

Integer frequencies between 0 and 30000000 with no units or punctuation

and #steps 4096 maximum

With no parameters, SWCENTER runs default settings of a center 1 MHz frequency and a 100 kHz sweep width, so the default sweep range is between 950 kHz and 1150 kHz.

The numeric keypad allows fine and coarse changes of center frequency and number of steps.

‘7’	raises the center frequency by 10%

‘1’	decreases the center frequency by 11%

‘9’	doubles the sweep frequency range

‘3’	halves the sweep frequency range

‘8’	halves the number of steps

‘2’	doubles the number of steps with an upper limit of 4096

‘4’	decreases the center frequency by the frequency step count for adjustable fine resolution

‘6’	increases the center frequency by the frequency step count for adjustable fine resolution

‘5’	increases the number of steps by 1/16th for adjustable fine resolution

Only these keys change operating parameters.  The reasoning behind ‘5’ was to provide fine control of the total sweep time in order to adjust it to an oscilloscope timebase for stable display.

When running, the program displays

- the center frequency,

- the total sweep width,

- the start frequency,

- the end frequency,

- the number of steps,

- the incremental DDS step count,

- the corresponding incremental frequency change,

- the instantaneous frequency,

- the incremental D/A Converter step count, and

- the instantaneous word the program writes to the D/A Converter

SSW.EXE

SSW starts from a command line only.  With no parameters, SSW prints a usage statement to explain the parameters it requires.

Usage: SSW filename.txt [start] [end] [#steps]

Enter parameters as integer Hertz with no punctuation or units.

Defaults are 1MHz, 10MHz, and 4096 steps

You must enter an output filename as the first parameter.  If you enter anything else, a text file appears on your drive with that as the filename.  The file appears in the same folder as the executable unless you redirect it using the old DOS filename rules.

With no other parameters, SSW runs default settings that sweep from 1 MHz to 10 MHz in 4096 equal steps.  If you follow the filename in the command line with only a single parameter less than 10 MHz, the DDS will sweep from that start frequency up to 10 MHz.  With additional parameters, you control the end frequency and number of steps.  The upper limit to the number of steps corresponds to an increment of one count in the 32 bit number written to the DDS, a very tiny fraction of a Hertz.

All numeric entries are decimal integer variables, so you must enter 5 MHz as 5000000 with no units or commas or spaces or any other punctuation.

The numeric keypad serves no function in this program.  There is only a single sweep with a measurement and record of data at each frequency step.

When running, the program displays

- the start frequency,

- the end frequency,

- the number of steps,

- the incremental DDS step count,

- the frequency increment,

- the instantaneous frequency, and

- the measured power from the log detector at Input 2

The program also writes the instantaneous frequency and measured power to the output text file.

LSW.EXE

LSW starts from a command line only.  With no parameters, SSW prints a usage statement to explain the parameters it requires.

Usage: LSW filename.txt [start] [end] step-fraction-denominator]

Enter parameters as integer Hertz with no punctuation or units.

Defaults are 1MHz, 10MHz, and 20, so each step is 5% higher in frequency

You must enter an output filename as the first parameter.  If you enter anything else, a text file appears on your drive with that as the filename.  The file appears in the same folder as the executable unless you redirect it using the old DOS filename rules.

With no other parameters, LSW runs default settings that sweep from 1 MHz to 10 MHz in increasingly larger steps, proportional to the frequency.  If you follow the filename in the command line with only a single parameter less than 10 MHz, the DDS will sweep from that start frequency up to 10 MHz.  With additional parameters, you control the end frequency and the step ratio.  The lower limit to the step ratio corresponds to an increment of one count in the 32 bit number written to the DDS, a very tiny fraction of a Hertz.

All numeric entries are decimal integer variables, so you must enter 5 MHz as 5000000 with no units or commas or spaces or any other punctuation.

The numeric keypad serves no function in this program.  There is only a single sweep with a measurement and record of data at each frequency step.

When running, the program displays

- the start frequency,

- the end frequency,

- the percent frequency change,

- the instantaneous frequency, and

- the measured power from the log detector at Input 2

The program also writes the instantaneous frequency and measured power to the output text file.

TRANSFER.EXE

TRANSFER starts from a command line only.  With no parameters, SSW prints a usage statement to explain the parameters it requires.

Usage: TRANSFER filename.txt [Vstart] [Vend] [#steps]

Vstart less than Vend and both between -2470 to +2470 mV

in integer mV with no decimal or units

You must enter an output filename as the first parameter.  If you enter anything else, a text file appears on your drive with that as the filename.  The file appears in the same folder as the executable unless you redirect it using the old DOS filename rules.

With no other parameters, TRANSFER runs default settings that sweep from the full range of the D/A Converter in 4096 equal steps from about -2.5 V to +2.5V.  If you follow the filename in the command line with only a single parameter less than 2500, the DDS will sweep from that start voltage up to +2.5 V.  With additional parameters, you control the end Voltage and number of steps.  The upper limit to the number of steps corresponds to an increment of one count to the 10 bit D/A Converter.

All numeric entries are decimal integer variables, so you must enter -0.42 V as 420 with no units or commas or spaces or any other punctuation.  There are additional error messages if the specified parameters lie outside of the allowed range or would cause a negative going sweep.

The numeric keypad serves no function in this program.  There is only a single sweep with a measurement and record of data at each voltage step.

When running, the program displays

- the decimal start and end numbers written to the D/A Converter,

- the start voltage

- the end voltage,

- the number of steps,

- the incremental D/A Converter step count,

- the instantaneous source voltage, and

- the measured voltage at Input 1

The program also writes the instantaneous source voltage and measured voltage to the output text file.

All programs contain the GNU Public License Statement:

This program is free software: you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation, either version 3 of the License, or (at your option) any later version.

This program is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the GNU General Public License for more details.

You should have received a copy of the GNU General Public License along with this program.  If not, see http://www.gnu.org/licenses/.

Double click this icon to view the GNU Public License.